--------------------------------------------------------------------------------

--

--   Traffic light Controller Benchmark -- Simulation vector documentation

--

-- Source: The model and test patterns were obtained from the Hardware C

--         model written by R. Gupta (7/23/90) at Stanford.

--

-- author: Champaka Ramachandran

--         University Of California, Irvine, CA 92717

--         champaka@balboa.eng.uci.edu

--

-- Written on August 11th 1992

--

--------------------------------------------------------------------------------

------------------

Testing Strategy :

------------------

There exists 4 Tests that cross check the following sequence of events :

Strategy 1

----------

Init --> Cars and TimeoutL --> TimeoutS --> !Cars  --> TimeoutS

-- Pattern #0

Cars <= '1';

TimeoutL <= '1';

TimeoutS <= '0';

-- Pattern #1

Cars <= '0';

TimeoutL <= '0';

TimeoutS <= '1';

-- Pattern #2

Cars <= '1';

TimeoutL <= '0';

TimeoutS <= '1';

-- Pattern #3

Cars <= '0';

TimeoutL <= '0';

TimeoutS <= '0';

-- Pattern #4

Cars <= '0';

TimeoutL <= '0';

TimeoutS <= '1';

1. Checks whether the initial condition sets the Highway

light to green and the Farm light to red.

2. Checks if there is a car on the side road and the long

timeout occurs, the highway light changes to yellow.

3. After a short time out the highway light changes to

red and the farm light changes to green.

4. If there are no cars on the side road, the farm light

changes to yellow.

5. After a short timeout, the farm light changes to red

and the highway light changes to green.

Strategy 2

----------

Cars and TimeoutL --> !Cars --> TimeoutL --> Timeout L and TimeoutS

-- Pattern #5

Cars <= '1';

TimeoutL <= '1';

TimeoutS <= '0';

-- Pattern #6

Cars <= '0';

TimeoutL <= '0';

TimeoutS <= '0';

-- Pattern #7

Cars <= '0';

TimeoutL <= '1';

TimeoutS <= '0';

-- Pattern #8

Cars <= '0';

TimeoutL <= '1';

TimeoutS <= '1';

1. Checks if there is a car on the side road and the long

timeout occurs, the highway light changes to yellow.

2. If there is no short timeout, then the Highway light

changes to red and the farm light changes to yellow.

3. No change

4. After a short timeout, the highway light changes to green

and the farm light changes to red.

Strategy 3

----------

Cars -> Cars & TimeoutS --> Cars & TimeoutL --> Cars, TimeoutS & Timeout L

--> Cars --> Cars & TimeoutL --> Cars and TimeoutS

-- Pattern #9

Cars <= '1';

TimeoutL <= '0';

TimeoutS <= '0';

-- Pattern #10

Cars <= '1';

TimeoutL <= '0';

TimeoutS <= '1';

-- Pattern #11

Cars <= '1';

TimeoutL <= '1';

TimeoutS <= '0';

-- Pattern #12

Cars <= '1';

TimeoutL <= '1';

TimeoutS <= '1';

-- Pattern #13

Cars <= '1';

TimeoutL <= '0';

TimeoutS <= '0';

-- Pattern #14

Cars <= '1';

TimeoutL <= '1';

TimeoutS <= '0';

-- Pattern #15

Cars <= '1';

TimeoutL <= '0';

TimeoutS <= '1';

1. No Change.

2. No Change.

3. Checks if there is a car on the side road and the long

timeout occurs, the highway light changes to yellow.

4. After a short time out the highway light changes to

red and the farm light changes to green.

5. No change.

6. If there is a long time out, the farm light changes to yellow.

7. After a short timeout, the highway light changes to green

and the farm light changes to red.

8. No Change.

Strategy 4

----------

-- Pattern #16

Cars <= '1';

TimeoutL <= '0';

TimeoutS <= '0';

-- Pattern #17

Cars <= '1';

TimeoutL <= '0';

TimeoutS <= '1';

wait for 10 ns;

-- Pattern #18

Cars <= '1';

TimeoutL <= '1';

TimeoutS <= '0';

-- Pattern #19

Cars <= '1';

TimeoutL <= '1';

TimeoutS <= '1';

-- Pattern #20

Cars <= '1';

TimeoutL <= '0';

TimeoutS <= '0';

-- Pattern #21

Cars <= '0';

TimeoutL <= '0';

TimeoutS <= '0';

-- Pattern #22

Cars <= '0';

TimeoutL <= '0';

TimeoutS <= '0';

-- Pattern #23

Cars <= '0';

TimeoutL <= '0';

TimeoutS <= '1';

Cars & TimeoutL --> Cars, TimeoutL & TimeoutS --> Cars --> None --> TimeoutS

1. Checks if there is a car on the side road and the long

timeout occurs, the highway light changes to yellow.

2. After a short time out the highway light changes to

red and the farm light changes to green.

3. No change.

4. If there are no cars on the side road, the farm light

changes to yellow.

5. After a short timeout, the highway light changes to green

and the farm light changes to red.